###############################################################################

# SWITCH.CFG - SAMPLE CONFIG FILE FOR MONITORING A SWITCH

#

#

# NOTES: This config file assumes that you are using the sample configuration

#    files that get installed with the Nagios quickstart guide.

#

###############################################################################

###############################################################################

#

# HOST DEFINITIONS

#

###############################################################################

# Define the switch that we'll be monitoring

define host {

use                     generic-switch                      ; Inherit default values from a template

host_name               linksys-srw224p                     ; The name we're giving to this switch

alias                   Linksys SRW224P Switch              ; A longer name associated with the switch

address                 192.168.1.253                       ; IP address of the switch

hostgroups              switches                            ; Host groups this switch is associated with

}

###############################################################################

#

# HOST GROUP DEFINITIONS

#

###############################################################################

# Create a new hostgroup for switches

define hostgroup {

hostgroup_name          switches                            ; The name of the hostgroup

alias                   Network Switches                    ; Long name of the group

}

###############################################################################

#

# SERVICE DEFINITIONS

#

###############################################################################

# Create a service to PING to switch

define service {

use                     generic-service                     ; Inherit values from a template

host_name               linksys-srw224p                     ; The name of the host the service is associated with

service_description     PING                                ; The service description

check_command           check_ping!200.0,20%!600.0,60%      ; The command used to monitor the service

check_interval          5                                   ; Check the service every 5 minutes under normal conditions

retry_interval          1                                   ; Re-check the service every minute until its final/hard state is determined

}

# Monitor uptime via SNMP

define service {

use                     generic-service                     ; Inherit values from a template

host_name               linksys-srw224p

service_description     Uptime

check_command           check_snmp!-C public -o sysUpTime.0

}

# Monitor Port 1 status via SNMP

define service {

use                     generic-service                     ; Inherit values from a template

host_name               linksys-srw224p

service_description     Port 1 Link Status

check_command           check_snmp!-C public -o ifOperStatus.1 -r 1 -m RFC1213-MIB

}

# Monitor bandwidth via MRTG logs

define service {

use                     generic-service                     ; Inherit values from a template

host_name               linksys-srw224p

service_description     Port 1 Bandwidth Usage

check_command           check_local_mrtgtraf!/var/lib/mrtg/192.168.1.253_1.log!AVG!1000000,1000000!5000000,5000000!10

}